Case Report Forms

TONIC VIBRATION REFLEX CHARACTERIZATION AND APPLICATION FOR EMG CONTROL

# Case Report Forms

| Location | Imperial College London |
| --- | --- |
| Tested By | Xinyao Wu |
| Supervised by | Mr. Patrick Sagastegui Alva, Prof. Dario Farina |

# Preliminary information

# Checklist

| Fulfilment of inclusion/exclusion criteria |  |
| --- | --- |
| Participant information sheet |  |
| Consent form |  |
| Measurement arm diameter |  |
| Photo of the setup |  |
| Connect MCU |  |
| Connect Bracelet |  |
| Connect Force sensor if necessary |  |
| Universal tactor controller + C-2 HDFL Tactor |  |
| Alcohol wipes and paper towels |  |
| External Monitor |  |

| Personal INFORMATION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Gender | male female |  |  |  |  |
| Date of birth: | __ __ /__ __ / __ __ |  |  |  |  |
| Dominant Hand | right left |  |  |  |  |
| Are there any relevant diseases? (stroke, cancer, surgeries,…) |  | Yes |  | No |  |
| If yes, please specify: |  |  |  |  |  |
| Is the participant currently taking any medication? |  |  | Yes |  | No |
| If yes, please specify (include dose): |  |  |  |  |  |

General Comments

Comments regarding the electrode position, stimulator placement: Comments on tasks or testing: Measurement of Arm (Diameter & length): Optimal frequency: 120Hz

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

PREREQUISITE (~5 minutes)

- Ask the dominant arm, age

- Clean forearm and EMG with alcohol wipes and dry with paper towels

- Locate flexor common tendon – Place stimulator on top of tendon – Indicate position on comments

- Set up EMG as in figure of general comments – Electrode 4 on top of the radial area

- If EMG loose, use medical gauze to increase contact with the skin

- Write subject ID

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SETUP (~5 minutes)

- Connect distribution power to MCU and stimulator controller

- Connect Stimulator by USB to computer

- Connect Stimulator to 1st port of stimulator controller and force sensor to 2nd input of MCU (Vcc-3V)

- Connect EMG to MCU

- Connect battery and turn on MCU

CALIBRATION – MEASUREMENT OF MAXIMAL VOLUNTARY CONTRACTION (~2 min) ------------------------------------------------------

Common to both days before starting the experiments.

- Ask the subject to release and rest their palms over the force sensors.

- Check EMG signal out of noises – without and with stimulation

- Activate PlotRadial EMG: Visually inspect which channel generates the higher activity

- Press ‘MVC’ at target %

- Indicate subject to emulate movements in the arm

- Select EMG channel feedback corresponding to the highest EMG amplitude. Chan [L|R]: _____

Time: (3s right + 10s rest + 3s left) x 2 repetitions + 20s rest = 1.3 min

PHYSIOLOGICAL (TRAINING) [10 min] ------------------------------------------------------------------------------------------------------------------------------------

- Enable visual feedback – Feedback: Individual

- Set frequency to 0

- Press ‘Start’

- 5 times training with visual feedback

- 5 times training asking the subject to close eyes after 5 seconds. Guide the subject verbally to maintain the corresponding force for the rest of the trial

Time: (5s reach target + 20s trial + 20s rest) x 20 repetitions x 3 strap. force = 45 min

PHYSIOLOGICAL (BLOCK I-A) – STRAP. FORCE RESPONSE [45 min] -------------------------------------------------------------------------------------

Do 5 trials for each considered stimulation amplitude

- Disable visual feedback – Feedback: Blind

- Set the optimal frequency and %MVC

- Define strapping force – pressure: 2 (low), 7 (medium), 12 (high) levels

- Make sure force sensor is ON

- Press ‘Start’

- 5 repetitions each pressure (20 reps)

- Activate PlotRadial EMG – during trials visually inspect which channel produce higher AEMG

- Do offline analysis - Select the channel with the higher AEMG: ____

Time: (5s reach target + 20s trial + 20s rest) x 20 repetitions x 2 materials = 30 min

PHYSIOLOGICAL (BLOCK I-B) – STRAP. FORCE FLUCTUATION REDUCTION [30 min] ----------------------------------------------------------

Do 5 trials for each considered stimulation amplitude

- Disable visual feedback – Feedback: Blind

- Set the optimal frequency, %MVC, strap. Force = 2N

- Press ‘Start’

- 5 repetitions each pressure (20 reps)

- Activate PlotRadial EMG – during trials visually inspect which channel produce higher AEMG

- Do offline analysis - Select the channel with the higher AEMG: ____

Time: (5s reach target + 20s trial + 5s rest) x 20 repetitions x 4 %MVC = 60 min

PHYSIOLOGICAL (BLOCK II) - AMPLITUDE & %MVC RESPONSE [60 min] ------------------------------------------------------------------------------

Do 5 trials for each considered stimulation amplitude

- Disable visual feedback – Feedback: Blind

- Set Strapping force to 2N, contact point at 9mm

- Define amplitudes – Amplitudes: zero-low-medium-high in the range of 0-255 at 120Hz.

- Experiment conducted under %MVC at 0%, 10%, 20%, 30% MVC

- Press ‘Start’

- 5 repetitions each amplitude (30 reps)

- Activate PlotRadial EMG – during trials visually inspect which channel produce higher AEMG

- Do offline analysis - Select the channel with the higher AEMG: ____

Time: (5s reach target + 20s trial + 5s rest) x 20 repetitions x 2 contact area diameter = 30 min

PHYSIOLOGICAL (BLOCK III-A) – CONTACT AREA RESPONSE [30 min] --------------------------------------------------------------------------------

Do 5 trials for each considered stimulation amplitude

- Disable visual feedback – Feedback: Blind

- Set the optimal frequency = 120Hz, %MVC = 10%, strap. Force = 2N

- Define contact point – higher diameter contact point: 9, 15mm

- Press ‘Start’

- 5 repetitions each pressure (30 reps)

- Activate PlotRadial EMG – during trials visually inspect which channel produce higher AEMG

- Do offline analysis - Select the channel with the higher AEMG: ____

Time: (5s reach target + 20s trial + 5s rest) x 20 repetitions x 2 models = 30 min

PHYSIOLOGICAL (BLOCK III-B) – BI-STIMULATOR PILOT TEST [30 min] -------------------------------------------------------------------------------

Do 5 trials for each considered stimulation amplitude

- Disable visual feedback – Feedback: Blind

- Set the optimal frequency = 120Hz, %MVC = 10%, strap. Force = 2N, contact area diameter = 9cm

- Define the second position of the stimulator proximal to the first.

- Press ‘Start’

- 5 repetitions each pressure (20 reps)

- Activate PlotRadial EMG – during trials visually inspect which channel produce higher AEMG

- Do offline analysis - Select the channel with the higher AEMG: ____

Time: (5s reach target + 20s trial + 5s rest) x 20 repetitions x 3 models = 45 min

PHYSIOLOGICAL (BLOCK III-C) – BI-STIMULATOR MODEL [45 min] --------------------------------------------------------------------------------

Do 5 trials for each considered stimulation amplitude

- Disable visual feedback – Feedback: Blind

- Set the optimal frequency = 120Hz, %MVC = 10%, strap. Force = 2N, contact area diameter = 9cm

- Define 3 bi-stimulator models: Single Proximal, Dual Proximal, Proximal+Distal

- Press ‘Start’

- 5 repetitions each pressure (30 reps)

- Activate PlotRadial EMG – during trials visually inspect which channel produce higher AEMG

- Do offline analysis - Select the channel with the higher AEMG: ____

AFTER EXPERIMENT -----------------------------------------------------------------------------------------------------------------------------------------------------

Take pictures of:

- EMG and stimulator on the arm – try to capture in the picture the EMG channels distribution around the forearm

- Marks left by the stimulator and EMG around the forearm – for precise location